AnyLink Cloud MQTT Data Access Protocol

Version: 1.2.0

Status: Controlled document

Copyright

Copyright of this document belongs to Jiangsu Ziqing Information Technology Co., Ltd. Anyone who copies or republishes this document without the company’s permission shall bear legal liability.

Contact us

Jiangsu Ziqing Information Technology Co., Ltd. always aims to provide customers with the most timely and comprehensive service. For any assistance, please contact us:

Jiangsu Ziqing Information Technology Co., Ltd.

Address: Room 1106, Beichuang Science Park, No. 401 Xingyuan North Road, Liangxi District, Wuxi, Jiangsu, China

Tel: 400-066-9060 / +86 0510-82602929

Email: marketing@anylink.io

For technical support or to report issues in this technical document, please send email to: support@anylink.io

Document revision history

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0.0 | 2024/08 |  | Initial version |
| 1.0.1 | 2024/10 | Song Yuping | Added itemId to control commands |
| 1.0.2 | 2024/12 | Yang Bingzhou | Added tag parameters to device OTA commands |
| 1.0.3 | 2025/02 | Song Yuping | Control command to upload all data once |
| 1.0.4 | 2025/03 | Song Yuping | Point-table registration supports packetized upload Added registration response |
| 1.0.5 | 2025/03 | Song Yuping | Added checksum to the registration file Added device heartbeat |
| 1.1.0 | 2026/03 | Luo Shuai | Added point-table config download, gateway OTA, and MQTT configuration |
| 1.2.0 | 2026/05 | Luo Shuai | Added data compression notes (Appendix 5) |

Contents

Terms1

1 MQTT information2

2 Data exchange3

2.1 Data-point registration3

2.1.1 Standard JSON registration3

2.1.2 Packetized registration5

2.1.3 Registration response8

2.2 Gateway attributes8

2.3 Device attributes9

2.4 Data upload10

2.5 Alarms11

2.6 Device heartbeat12

2.7 Device online/offline13

2.8 Data control13

2.8.1 Downlink control command13

2.8.2 Uplink control result14

2.9 OTA (device OTA)15

2.9.1 Downlink OTA command15

2.9.2 Uplink OTA result16

2.10 OTA (gateway OTA)17

2.10.1 Downlink OTA command18

2.10.2 Uplink OTA result18

2.11 Force gateway to upload device data19

2.11.1 Platform downlink command19

2.11.2 Gateway response20

2.12 Downlink device point-table configuration20

2.12.1 Platform sends point-table configuration20

2.12.2 Gateway response23

2.13 Modify gateway MQTT configuration24

2.13.1 Platform downlink MQTT parameters24

2.13.2 Gateway response25

Appendix 1: item type27

Appendix 2: Control command types (cmdType)27

Appendix 3: Topic Code27

Appendix 4: Transform28

Appendix 5: LZ4 data compression29

Terms

- unique_id: Unique identifier of the data collector (gateway), similar to a serial-number barcode. Length must not exceed 16 characters.

- Server: the MQTT broker.

- Client: the MQTT client, generally the data collector (device). The MQTT client id format is THIRD_${unique_id}. A persistent connection must be used; establishing a new connection for every publish is not allowed.

- In this document ${string} means the value of that string. For example, when unique_id=12345, topic /third/standard/up/${unique_id}/data means /third/standard/up/12345/data

- MQTT information

Required MQTT information includes: server address, port, MQTT username, and MQTT password.

Obtain the above information from the cloud platform vendor.

- Data exchange

  - Data-point registration

2.1.1 Standard JSON registration

Note: Whenever any data-point information configured in the gateway changes, the configuration must be sent again to this topic.

Topic：/third/standard/up/${unique_id}/config

JSON body:

{ "dataCode": "deviceRegister", "data": { "checksum": "e10adc3949b", // Optional. Registration file id (max 32 chars), computed from data excluding checksum and timestamp. Same checksum means the file is unchanged; otherwise it has changed "uniqueId": "1234567", // Collector serial number: digits and English letters, length <= 16 "timestamp": 1474964078659, // UNIX timestamp (ms) "name": "Flow meter monitoring", "devices": [ // Device monitoring configuration { "id": "abc", // Device ID: digits and English letters, length <= 20 "name": "Flow meter", // Device name "items": [ { "id": 1, // Data-item id, unique within the current device "name": "FlowTheDay", // Data-point name, unique within the current device "alias": "Flow", // Data-point alias "type": 2, // Data type, see Appendix 1 "frequency": 1000, // Collection interval, ms "writable": 0, // Access: 0=read-only, 1=read/write "unit": "m" // Unit }, { "id": 2, "alias": "Switch", "name": "Switch", "type": 2, "frequency": 1000, "writable": 0, "unit": "m" } ], "rules": [ { "id": 1, // Id, unique within the current device "name": "Alarm rule name", // Alarm rule name "severity": 8, // Alarm severity 8, 9, 10, 11, 12 "description": "Alarm description", // Alarm description "items": [ { "id": 1, // Data-item id associated with the alarm rule "operator": "<,<", // Comparison operator "operand1": "10", // Threshold 1 "operand2": "80" // Threshold 2 } ] } ] } ] } }

* Alarm severity:

| Value | Meaning |
| --- | --- |
| 8 | Level 1 |
| 9 | Level 2 |
| 10 | Level 3 |
| 11 | Level 4 |
| 12 | Notification |

* Comparison operators:

In the examples below, L1 is threshold 1, L2 is threshold 2, and value is the associated data-item value.

| Value | Meaning |
| --- | --- |
| == | value == L1 |
| != | value != L1 |
| > | value > L1 |
| >= | value >= L1 |
| < | value < L1 |
| <= | value <= L1 |
| <,< | L1 < value < L2 |
| <=,< | L1 <= value < L2 |
| <,<= | L1 < value <= L2 |
| <=,<= | L1 <= value <= L2 |

2.1.2 Packetized registration

When gateway limits prevent uploading a complete JSON registration in one message, packetized upload may be used: split the JSON into several parts, prepend packet headers, and upload sequentially.

Note: the timestamp in the header must be the same for all packets of one registration. Packets with the same timestamp are assembled into one complete registration.

- Topic：/third/standard/up/packet/${unique_id}/config

Uploaded data format: packet header + byte array of the split JSON body + CRC16

- Data layout:

File type (1 byte, currently 0x01) +

Total packet count (2 bytes, little-endian) +

Current packet number (2 bytes, starting from 1, little-endian) +

Timestamp (6 bytes: first 4 bytes = seconds, last 2 bytes = milliseconds) +

Body (byte array of the split JSON) +

Checksum (2 bytes, Modbus CRC16 over all preceding bytes)

Example: 0x01,0x03,0x00,0x01,0x00,0x88,0x13,0xD9,0x67,0XE7,0x03,body,CRC

Note: 0x01 file type

0x03,0x00 total packets: 3

0x01,0x00 current packet number: 1

0x88,0x13,0xD9,0x67 first 4 timestamp bytes, seconds = 1742279560

0XE7,0x03 last 2 timestamp bytes, milliseconds = 999

Body: convert the original JSON registration file to a binary byte array, then split by the packet size supported by the hardware

Example JSON registration file:

{

"dataCode": "deviceRegister",

"data": {

"uniqueId": "1234567",

"timestamp": 1474964078659,

"name": "Flow meter monitoring",

"devices": [

{

"id": "abc",

"name": "Flow meter",

"items": [

{

"id": 1,

"name": "FlowTheDay",

"alias": "Flow",

"type": 2,

"frequency": 1000,

"writable": 0,

"unit": "m"

},

{

"id": 2,

"alias": "Switch",

"name": "Switch",

"type": 2,

"frequency": 1000,

"writable": 0,

"unit": "m"

}

],

"rules": [

{

"id": 1,

"name": "Alarm rule name",

"severity": 8,

"description": "Alarm description",

"items": [

{

"id": 1,

"operator": "<,<",

"operand1": "10",

"operand2": "80"

}

]

}

]

}

]

}

}

{

"dataCode": "deviceRegister",

"data": {

"uniqueId": "1234567",

"timestamp": 1474964078659,

"name": "Flow meter monitoring",

"devices": [

{

"id": "abc",

"name": "Flow meter",

"items": [

{

"id": 1,

"name": "FlowTheDay",

"alias": "Flow",

"type": 2,

"frequency": 1000,

"writable": 0,

"unit": "m"

},

{

"id": 2,

"alias": "Switch",

"name": "Switch",

"type": 2,

"frequency": 1000,

"writable": 0,

"unit": "m"

}

],

"rules": [

{

"id": 1,

"name": "Alarm rule name",

"severity": 8,

"description": "Alarm description",

"items": [

{

"id": 1,

"operator": "<,<",

"operand1": "10",

"operand2": "80"

}

]

}

]

}

]

}

}

Corresponding body after conversion to a byte array:

0x7b,0x22,0x64,0x61,0x74,0x61,0x43,0x6f,0x64,0x65,0x22,0x3a,0x22,0x64,0x65,0x76,0x69,0x63,0x65,0x52,0x65,0x67,0x69,0x73,0x74,0x65,0x72,0x22,0x2c,0x22,0x64,0x61,0x74,0x61,0x22,0x3a,0x7b,0x22,0x75,0x6e,0x69,0x71,0x75,0x65,0x49,0x64,0x22,0x3a,0x22,0x31,0x32,0x33,0x34,0x35,0x36,0x37,0x22,0x2c,0x22,0x74,0x69,0x6d,0x65,0x73,0x74,0x61,0x6d,0x70,0x22,0x3a,0x31,0x34,0x37,0x34,0x39,0x36,0x34,0x30,0x37,0x38,0x36,0x35,0x39,0x2c,0x22,0x6e,0x61,0x6d,0x65,0x22,0x3a,0x22,0xe6,0xb5,0x81,0xe9,0x87,0x8f,0xe8,0xae,0xa1,0xe7,0x9b,0x91,0xe6,0x8e,0xa7,0x22,0x2c,0x22,0x64,0x65,0x76,0x69,0x63,0x65,0x73,0x22,0x3a,0x5b,0x7b,0x22,0x69,0x64,0x22,0x3a,0x22,0x61,0x62,0x63,0x22,0x2c,0x22,0x6e,0x61,0x6d,0x65,0x22,0x3a,0x22,0xe6,0xb5,0x81,0xe9,0x87,0x8f,0xe8,0xae,0xa1,0x22,0x2c,0x22,0x69,0x74,0x65,0x6d,0x73,0x22,0x3a,0x5b,0x7b,0x22,0x69,0x64,0x22,0x3a,0x31,0x2c,0x22,0x6e,0x61,0x6d,0x65,0x22,0x3a,0x22,0x46,0x6c,0x6f,0x77,0x54,0x68,0x65,0x44,0x61,0x79,0x22,0x2c,0x22,0x61,0x6c,0x69,0x61,0x73,0x22,0x3a,0x22,0xe6,0xb5,0x81,0xe9,0x87,0x8f,0x22,0x2c,0x22,0x74,0x79,0x70,0x65,0x22,0x3a,0x32,0x2c,0x22,0x66,0x72,0x65,0x71,0x75,0x65,0x6e,0x63,0x79,0x22,0x3a,0x31,0x30,0x30,0x30,0x2c,0x22,0x77,0x72,0x69,0x74,0x61,0x62,0x6c,0x65,0x22,0x3a,0x30,0x2c,0x22,0x75,0x6e,0x69,0x74,0x22,0x3a,0x22,0x6d,0x22,0x7d,0x2c,0x7b,0x22,0x69,0x64,0x22,0x3a,0x32,0x2c,0x22,0x61,0x6c,0x69,0x61,0x73,0x22,0x3a,0x22,0xe5,0xbc,0x80,0xe5,0x85,0xb3,0x22,0x2c,0x22,0x6e,0x61,0x6d,0x65,0x22,0x3a,0x22,0x53,0x77,0x69,0x74,0x63,0x68,0x22,0x2c,0x22,0x74,0x79,0x70,0x65,0x22,0x3a,0x32,0x2c,0x22,0x66,0x72,0x65,0x71,0x75,0x65,0x6e,0x63,0x79,0x22,0x3a,0x31,0x30,0x30,0x30,0x2c,0x22,0x77,0x72,0x69,0x74,0x61,0x62,0x6c,0x65,0x22,0x3a,0x30,0x2c,0x22,0x75,0x6e,0x69,0x74,0x22,0x3a,0x22,0x6d,0x22,0x7d,0x5d,0x2c,0x22,0x72,0x75,0x6c,0x65,0x73,0x22,0x3a,0x5b,0x7b,0x22,0x69,0x64,0x22,0x3a,0x31,0x2c,0x22,0x6e,0x61,0x6d,0x65,0x22,0x3a,0x22,0xe5,0x91,0x8a,0xe8,0xad,0xa6,0xe8,0xa7,0x84,0xe5,0x88,0x99,0xe5,0x90,0x8d,0xe7,0xa7,0xb0,0x22,0x2c,0x22,0x73,0x65,0x76,0x65,0x72,0x69,0x74,0x79,0x22,0x3a,0x38,0x2c,0x22,0x64,0x65,0x73,0x63,0x72,0x69,0x70,0x74,0x69,0x6f,0x6e,0x22,0x3a,0x22,0xe5,0x91,0x8a,0xe8,0xad,0xa6,0xe6,0x8f,0x8f,0xe8,0xbf,0xb0,0x22,0x2c,0x22,0x69,0x74,0x65,0x6d,0x73,0x22,0x3a,0x5b,0x7b,0x22,0x69,0x64,0x22,0x3a,0x31,0x2c,0x22,0x6f,0x70,0x72,0x61,0x74,0x6f,0x72,0x22,0x3a,0x22,0x3c,0x2c,0x3c,0x22,0x2c,0x22,0x6f,0x70,0x65,0x72,0x61,0x6e,0x64,0x31,0x22,0x3a,0x22,0x31,0x30,0x22,0x2c,0x22,0x6f,0x70,0x65,0x72,0x61,0x6e,0x64,0x32,0x22,0x3a,0x22,0x38,0x30,0x22,0x7d,0x5d,0x7d,0x5d,0x7d,0x5d,0x7d,0x7d

Packetization splits the byte array above.

A complete packet after adding the header:

0x01,0x03,0x00,0x01,0x00,0x88,0x13,0xD9,0x67,0XE7,0x03,0x7b,0x22,0x64,0x61,0x74,0x61,0x43,0x6f,0x64,0x65,0x22,0x3a,0x22,0x64,0x65,0x76,0x69,0x63,0x65,0x52,0x65,0x67,0x69,0x73,0x74,0x65…

Yellow text = header, gray text = timestamp, blue text = split body.

2.1.3 Registration response

After the server receives a complete data-point registration, it parses and stores the content, then pushes the result to the gateway.

- Topic: /third/standard/down/${unique_id}/config/resp

Registration response format:

{ "result_code": "error code", "result_message": "error description", "timestamp": 1474964078659 // UNIX timestamp (ms) }

- Registration response error codes

| Error code | Description |
| --- | --- |
| 200000 | Registration succeeded |
| 200001 | Failed to parse the configuration file |
| 500 | Internal server processing error |

  - Gateway attributes

After the gateway starts, it may upload gateway attributes to the platform (fields can be extended). This flow is optional.

- Topic：/third/standard/up/${unique_id}/attribute

JSON body:

{ "dataCode": "gatewayAttribute", "data": { "uniqueId": "1234567", "timestamp": 1474964078659, // UNIX timestamp (ms) "attributes": { "CSQ": "29", "simid": "898602123434343543323" } } }

The data field is a JSON object. Keys are attribute names and values are attribute values.

Allowed attributes:

| Attribute | Description |
| --- | --- |
| gateModel | Collector model |
| agentVersion | Collector firmware version |
| CSQ | Collector cellular / Wi-Fi signal strength |
| simid | SIM card ICCID |

  - Device attributes

After the gateway starts, it may upload gateway attributes to the platform (fields can be extended). This flow is optional.

- Topic：/third/standard/up/${unique_id}/attribute

JSON body:

{ "dataCode": "deviceAttribute", "data": { "uniqueId": "1234567", "timestamp": 1474964078659, // UNIX timestamp (ms) "devices": [ { "id": "123abc", "attributes": { "productName": "Integrated prefabricated pump", "productId": "00-E0-4C-36-0D-6D" } } ] } }

The data field is a JSON object. Keys are attribute names and values are attribute values.

Allowed attributes:

| Attribute | Description |
| --- | --- |
| device_alias | Device alias |
| productId | Product ID |
| productName | Product name |
| installPlace | Installation address |
| userName | User name |
| userPhone | User phone number |
| serialNumber | Gateway serial number |
| wxLocation | Geographic location |
| productModel | Product model |
| controllerModel | Controller model |

  - Data upload

Note: if LZ4 compression is used, see Appendix 5 for the topic

Topic: /third/standard/up/${unique_id}/data

JSON format:

{ "dataCode": "deviceItemData", "data": { "uniqueId": "1234567", "timestamp": 1651730442099, // UNIX timestamp (ms) "devices": [ { "id": "123456abc", "items": [ { "id": 1, "value": "0.100000001" }, { "id": 2, "value": "0.100000001" } ] } ] } }

  - Alarms

Note: if LZ4 compression is used, see Appendix 5 for the topic

Topic：/third/standard/up/${unique_id}/alarm

JSON format:

Alarm triggered:

{ "dataCode": "deviceAlarm", "data": { "uniqueId": "1234567", "timestamp": 1651730442099, // UNIX timestamp (ms) "active": true, // true = alarm triggered "devices": [ { "id": "123456abc", "alarms": [ { "ruleId": 1, // Alarm rule ID "items": [ { "id": 1, // Data-item id "value": "data-point value when the alarm occurred" } ] } ] } ] } }

Alarm cleared:

{ "dataCode": "deviceAlarm", "data": { "uniqueId": "1234567", "timestamp": 1651730442099, // UNIX timestamp (ms) "active": false, // false = alarm cleared "devices": [ { "id": "123456abc", "alarms": [ { "ruleId": 1, // Alarm rule ID "items": [ { "id": 1, // Data-item id "value": "data-point value when the alarm was cleared" } ] } ] } ] } }

  - Device heartbeat

Send a heartbeat every 5 minutes. If more than 1.5 heartbeats are missed, the device is considered offline.

Topic: /third/standard/up/${unique_id}/hearbeat

JSON format:

{

"dataCode": "heartbeat",

"data": {

"uniqueId": "1234567",

"timestamp": 1651730442099, // UNIX timestamp (ms)

"deviceIds": ["1", "2", "3"]

}

}

  - Device online / offline

Topic: /third/standard/up/${unique_id}/deviceState

JSON format:

{ "dataCode": "deviceState", "data": { "uniqueId": "1234567", "timestamp": 1651730442099, // UNIX timestamp (ms) "state": 0, "deviceIds": ["1", "2", "3"] } }

Note: state=1 means online, state=0 means offline

If the deviceIds array is empty, state applies to all devices. For example, state=0 means all devices are offline; state=1 means all devices are online.

  - Data control

2.8.1 Downlink control command

Topic：/third/standard/down/${unique_id}/control

Data format:

{

"dataCode": "deviceControl",

"data": {

"uniqueId": "1234567", // Gateway unique ID

"timestamp": 1474964078659, // UNIX timestamp (ms)

"messageId": "1651730442099", // User-defined value; the gateway must return it unchanged

"cmdType": 0, // Control command type, e.g. modify a data-item value or send a serial command (see Appendix 2)

"cmds": [

{

"deviceId": "123456abc",

"itemName": "FlowTheDay",

"itemId": 14,

"value": "1"

}

]

}

}

2.8.2 Uplink control result

Topic：/third/standard/up/${unique_id}/control/resp

JSON format:

{ "dataCode": "deviceControlResponse", "data": { "resultCode": "error code", "resultMessage": "error description", "messageId": "user-defined value, returned unchanged", "timestamp": 1474964078659 // UNIX timestamp (ms) } }

Error codes:

| Error code | Description |
| --- | --- |
| 0 | Command executed successfully |
| -1 | Command execution failed |
| 3 | Timeout |
| 5 | Unknown error |
| 6 | Command format error (e.g. the downlink command is not valid JSON) |

  - OTA (device OTA)

2.9.1 Downlink OTA command

Topic：/third/standard/down/${unique_id}/OTA

JSON format:

{ "dataCode": "deviceOTACmd", "data": { "uniqueId": "1234567", // Gateway unique ID "timestamp": 1474964078659, // UNIX timestamp (ms) "messageId": "1651730442099", // Must be returned unchanged in the gateway response "cmdType": 1002, // Control command type, see Appendix 2 "params": { "url": "http://test.anylink.io:88/ota/fefdsf.bin", // OTA file download URL "md5": "bd505abdf341b3a90bc00957", // OTA file MD5 "tag1": "", // OTA device protocol tag, optional "tag2": "N20", // Custom OTA tag, optional "tag3": "1234" // Custom OTA tag, optional } } }

2.9.2 Uplink OTA result

Topic：/third/standard/up/${unique_id}/OTA/resp

JSON format:

{ "dataCode": "deviceOTAResponse", "data": { "statusCode": 2, // Current OTA stage "resultCode": 100, // Result "resultMessage": "error description", "messageId": "user-defined value, returned unchanged", "timestamp": 1474964078659 // UNIX timestamp (ms) } }

statusCode and resultCode:

| statusCode | OTA stage | resultCode |
| --- | --- | --- |
| 1 | Whether the gateway is ready | - Gateway is ready for OTA - Upgrade forbidden on the gateway - Failed to get upgrade directory - Directory does not exist - Version check failed - Command validation failed - Command parse failed Extensible |
| 2 | File download | 100 — file downloaded and verified 101 — download failed 102 — file verification failed 103 — file receive timeout 104 — decompress failed Extensible |
| 3 | Upgrade result | 200 — upgrade succeeded 201 — upgrade failed Extensible |

  - OTA (gateway OTA)

2.10.1 Downlink OTA command

Topic：/third/standard/down/${unique_id}/OTA

JSON format:

{ "dataCode": "gatewayOTACmd", "data": { "uniqueId": "1234567", // Gateway unique ID "timestamp": 1474964078659, // UNIX timestamp (ms) "messageId": "1651730442099", // Must be returned unchanged in the gateway response "cmdType": 1001, // Control command type, see Appendix 2 "params": { "url": "http://test.anylink.io:88/ota/fefdsf.bin", // OTA file download URL "md5": "bd505abdf341b3a90bc00957" // OTA file MD5 } } }

2.10.2 Uplink OTA result

Topic：/third/standard/up/${unique_id}/OTA/resp

JSON format:

{ "dataCode": "gatewayOTAResponse", "data": { "statusCode": 2, // Current OTA stage "resultCode": 100, // Result "resultMessage": "error description", "messageId": "user-defined value, returned unchanged", "timestamp": 1474964078659 // UNIX timestamp (ms) } }

statusCode and resultCode:

| statusCode | OTA stage | resultCode |
| --- | --- | --- |
| 1 | Whether the gateway is ready | 0 -- gateway is ready for OTA 1 -- upgrade forbidden on the gateway 2 -- failed to get upgrade directory 3 -- directory does not exist 4 -- version check failed 5 -- command validation failed 6 -- command parse failed Extensible |
| 2 | File download | 100 — file downloaded and verified 101 — download failed 102 — file verification failed 103 — file receive timeout 104 — decompress failed Extensible |
| 3 | Upgrade result | 200 — upgrade succeeded 201 — upgrade failed Extensible |

  - Force the gateway to upload device data

2.11.1 Platform downlink command

Topic：/third/standard/down/${unique_id}/control

Data format:

{ "dataCode": "deviceControl", "data": { "uniqueId": "1234567", // Gateway unique ID "timestamp": 1474964078659, // UNIX timestamp (ms) "messageId": "1651730442099", // User-defined value; the gateway must return it unchanged "cmdType": 3, // Control command type, e.g. modify a data-item value or send a serial command (see Appendix 2) "cmds": [ { "shell": "UPLOAD_ITEMS_DATA", // Upload live data immediately "param": { "all": 1, // 1=all devices, 0=some devices, combined with items "items": [ { "all": 1, // 1=all data of this device, 0=partial data "deviceId": 1, "itemIds": [1, 2, 3] } ] } } ] } }

2.11.2 Gateway response

The gateway uploads the related data according to the control command, and returns the command result. See [2.8.2 Uplink control result].

  - Downlink device point-table configuration

2.12.1 Platform sends point-table configuration to the gateway

Topic: /third/standard/down/${unique_id}/config

Description: the platform sends a JSON packet containing the download URL of the point-table file. The gateway downloads the file from that URL.

Downlink JSON command:

{

"dataCode": "deviceConfig",

"data": {

"uniqueId": "1234567", // Gateway unique ID

"timestamp": 1474964078659, // UNIX timestamp (ms)

"messageId": "1651730442099", // Must be returned unchanged in the gateway response

"cmdType": 1000, // Command type, see Appendix 2

"params": {

"url": "http://test.anylink.io:88/config/N20_202501212.json", // Device point-table file download URL

"md5": "bd505abdf341b3a90bc00957" // OTA file MD5

}

}

}

Example point-table configuration file:

{ "name": "Flow meter monitoring", // Gateway name "devices": [ // Device monitoring configuration { "id": "abc", // Device ID: digits and English letters, length <= 20 "config": { "protocol": "modbus-rtu", // Protocol "connection": { "channel": "com1", // Port com1 ... comN "baudRate": 9600, // Baud rate 600~256000 "dataBits": 8, // Data bits 5/6/7/8 "stopBits": 1, // Stop bits 1/1.5/2 "parity": "none", // Parity none/odd/even "timeout": 1000 // Timeout (ms) } }, "name": "Flow meter", // Device name "items": [ { "id": 1, // Data-item id, unique within the current device "name": "FlowTheDay", // Data-point name, unique within the current device "alias": "Flow", // Data-point alias "type": 2, // Data type, see Appendix 1 "frequency": 1000, // Collection interval (ms) "writable": 0, // Access: 0=read-only, 1=read/write "unit": "m", // Unit "config": { "slave": 1, // Slave address "address": 40001, // Modbus address "code": 3, // Modbus function code "transform": "ADD(10)" // Data transform, see Appendix 4 } }, { "id": 2, "alias": "Switch", "name": "Switch", "type": 2, "frequency": 1000, "writable": 0, "unit": "m" } ], "rules": [ { "id": 1, // Id, unique within the current device "name": "Alarm rule name", // Alarm rule name "severity": 8, // Alarm severity 8, 9, 10, 11, 12 "description": "Alarm description", // Alarm description "items": [ { "id": 1, // Data-item id associated with the alarm rule "operator": "<,<", // Comparison operator "operand1": "10", // Threshold 1 "operand2": "80" // Threshold 2 } ] } ] } ] }

2.12.2 Gateway response

Topic: /third/standard/up/${unique_id}/config/resp

Description: the gateway returns the result of receiving the configuration file

JSON format:

{ "dataCode": "deviceConfigResponse", "data": { "resultCode": "error code", "resultMessage": "error description", "timestamp": 1474964078659 // UNIX timestamp (ms) } }

Error codes:

| Error code | Description |
| --- | --- |
| 100 | File received and verified |
| 101 | File receive failed |
| 102 | File format verification failed |

  - Modify gateway MQTT configuration

2.13.1 Platform downlink MQTT parameters

Topic：/third/standard/down/${unique_id}/mqtt

Description: if topics.subscribe and topics.publish are not configured, the gateway uses its default subscribe/publish rules.

Data format:

{

"dataCode": "mqttParams",

"data": {

"uniqueId": "1234567",

"timestamp": 1474964078659, // UNIX timestamp (ms)

"messageId": "1651730442099", // User-defined value; the gateway must return it unchanged

"cmdType": 4, // Control command type (see Appendix 2)

"cmds": [

{

"shell": "SET_MQTT_PARAMETERS", // Update MQTT configuration

"param": {

"mqtt_version": "3.1.1", // MQTT protocol version

"protocol": "mqtt/mqtts/ws/wss",

"ssl_verify": true, // Verify server identity (mutual TLS)

"tls_version": "TLSv1.2",

"host": "anylink.io",

"port": 1883,

"keepalive_interval_s": 60, // Seconds

"qos": 1,

"retained": false,

"client_id": "",

"username": "",

"password": "",

"clean_session": true,

"connect_timeout_ms": 10000, // Milliseconds

"reconnect": true,

"reconnect_period_ms": 4000, // Milliseconds

"topics": {

"subscribe": [

{

"code": "DEVICE_CTRL", // Topic function code (see Appendix 3)

"topic": "/third/standard/down/${unique_id}/control",

"qos": 0

}

],

"publish": [

{

"code": "DEVICE_CTRL_RESP",

"topic": "/third/standard/up/${unique_id}/control/resp",

"qos": 0,

"retain": false

}

]

}

}

}

]

}

}

2.13.2 Gateway response

Topic：/third/standard/up/${unique_id}/mqtt/resp

JSON format:

{ "dataCode": "mqttParams", "data": { "resultCode": "error code", "resultMessage": "error description", "messageId": "user-defined value, returned unchanged", "timestamp": 1474964078659 // UNIX timestamp (ms) } }

Error codes:

| Error code | Description |
| --- | --- |
| 0 | Command executed successfully |
| -1 | Command execution failed |
| 3 | Timeout |
| 5 | Unknown error |
| 6 | Command format error (e.g. the downlink command is not valid JSON) |

Appendix 1: item type

items.type in the JSON registration file:

| Type | Data type |
| --- | --- |
| 1 | Boolean |
| 2 | Byte |
| 3 | Word |
| 4 | DWord |
| 5 | float |
| 6 | double |
| 7 | int16 |
| 8 | int32 |

Appendix 2: Control command types (cmdType)

| Control command type | Description |
| --- | --- |
| 0 | Modify a data-point value |
| 2 | Send a serial-port command |
| 3 | Command: upload all data once |
| 4 | Command: modify gateway MQTT configuration |
| 1000 | Device point-table downlink |
| 1001 | OTA — gateway OTA |
| 1002 | OTA — device OTA |

Appendix 3: Topic Code

| topic code | Function |  |
| --- | --- | --- |
| REGISTER | Point-table registration | /third/standard/up/${unique_id}/config |
| REGISTER_PACKET | Packetized registration | /third/standard/up/packet/${unique_id}/config |
| REGISTER_RESP | Registration response | /third/standard/down/${unique_id}/config/resp |
| ATTR | Gateway attributes | /third/standard/up/${unique_id}/attribute |
| DEVICE_ATTR | Device attributes | /third/standard/up/${unique_id}/attribute |
| ITEM_DATA | Collected device data | /third/standard/up/${unique_id}/data |
| ALARM | Alarms | /third/standard/up/${unique_id}/alarm |
| HEARTBEAT | Device heartbeat | /third/standard/up/${unique_id}/hearbeat |
| DEVICE_STATE | Device online status | /third/standard/up/${unique_id}/deviceState |
| DEVICE_CTRL | Downlink control command | /third/standard/down/${unique_id}/control |
| DEVICE_CTRL_RESP | Control command result | /third/standard/up/${unique_id}/control/resp |
| OTA | OTA | /third/standard/down/${unique_id}/OTA |
| OTA_RESP | OTA response | /third/standard/up/${unique_id}/OTA/resp |
| CONFIG | Downlink device point-table configuration | /third/standard/down/${unique_id}/config |
| CONFIG_RESP | Configuration response | /third/standard/up/${unique_id}/config/resp |
| MQTT_PARAMS | Remote MQTT configuration | /third/standard/down/${unique_id}/mqtt |
| MQTT_PARAMS_RESP | MQTT configuration response | /third/standard/up/${unique_id}/mqtt/resp |

Appendix 4: Transform

| Transform | Description (x is the collected value) | Example (x=32) |
| --- | --- | --- |
| ADD(a) | Add: x+a | ADD(10) -> 32+10 -> 42 |
| SUB(a) | Subtract: x-a | SUB(10) -> 32-10 -> 22 |
| MUL(a) | Multiply: x·a | MUL(0.01) -> 32·0.01 -> 0.32 |
| DIV(a) | Divide: x/a (a!=0) | DIV(10) -> 32/10 -> 3.2 |
| LINEAR(a,b) | Linear: a·x+b | LINEAR(0.01,10) -> 32·0.01+10 -> 10.32 |
| BIT(a,b) | Bits: take bits a through b of x (b≥a; a,b are 0-based) | BIT(4,7) - 0010 0000 (binary of 32, bits 4-7 from left) -> 0010 -> 2 |

Appendix 5: LZ4 data compression

If the gateway uploads with LZ4 compression, device data and alarms are published on the topics below. The payload is LZ4-compressed before it is published to the MQTT broker.

| topic code | Function | Default topic |
| --- | --- | --- |
| ITEM_DATA | Collected device data | /third/standard/up/${unique_id}/data/lz4 |
| ALARM | Alarms | /third/standard/up/${unique_id}/alarm/lz4 |